[image: image2.jpg]HAFFA




 [image: image3.png]




Volume weight equivalent for the purpose of screening of KC cargo in prepacked unit

CLG had agreed with HAFFA to use the principle of 15% wastage to determine the volume weight equivalent.

 

Based on the ULD technical manual and 15% wastage, CLG has worked out the following volume weight equivalent of the most commonly used ULD for prepacked shipment. To keep it simple, irregular contour types (including winged pallet) are not included.

1.)
LD3
L 61.5N W 60.4N H 64N

Internal volume = 157 cu. ft.

Less 15% wastage = 133 cu. ft.

Use 1:1 density (366 cu.in. to 1 KG) = 133 cu. ft. x 1728 divided by 366 cu. in. = 628 KG

 

LD3 volume weight equivalent 
 

133 cu.ft. or 3.77 MC (628 KG)
 

 

2.) 
LD9 (88N) 

 

L 125N W 88N H 62N

Internal volume = 364 cu. ft.

Less 15% wastage = 309 cu. ft.

Use 1:1 density (366 cu.in. to 1 KG) = 309 cu. ft. x 1728 divided by 366 cu. in. = 1459 KG

 

LD9 (88N) volume weight equivalent 
 

309 cu.ft. or 8.75 MC (1459 KG)
 

 

3.) 
LD9 (96N) 

 

L 125N W 96N H 62N

Internal volume =  403 cu. ft.

Less 15% wastage =  343 cu. ft.

Use 1:1 density (366 cu.in. to 1 KG) =  343 cu. ft. x 1728 divided by 366 cu. in. = 1619 KG

 

LD9 (96N) volume weight equivalent
 

343 cu.ft. or 9.71 MC (1619 KG)
 

 

4.) 
Q6 

 

L 125N W 96N H 95.3N

Internal volume = 600 cu. ft.

Less 15% wastage = 510 cu. ft.

Use 1:1 density (366 cu.in. to 1 KG) = 510 cu. ft. x 1728 divided by 366 cu. in. = 2408 KG

 

Q6 volume weight equivalent
 

510 cu.ft. or 14.44 MC (2408 KG)
 

 

5.) 
Q7 

 

L 125N W 96N H 118N

Internal volume = 710 cu. ft.

Less 15% wastage = 603 cu. ft.

Use 1:1 density (366 cu.in. to 1 KG) = 603 cu. ft. x 1728 divided by 366 cu. in. = 2847 KG

 

Q7 volume weight equivalent
 


603 cu.ft. or 17.08 MC (2847 KG)







































[image: image1.jpg]HAFFA



[image: image2.jpg][image: image3.png]